Satellite Manoeuvre Detection

Technical Reference

orbit_analysis_kepler.py  ·  orbit_analysis_od.py

Overview

This document describes two independent Python modules for detecting satellite manoeuvres from publicly available orbital data. Both modules compare consecutive orbital states, flag anomalous changes, and cross-check detections against the Kp geomagnetic index to distinguish genuine manoeuvres from space-weather artefacts.

# 1.  Architecture Overview

Both modules share a common three-stage pipeline:

| Stage | Module | Description |
| --- | --- | --- |
| 1. Data Acquisition | Both | Download and cache orbital data (TLEs or element history) and the GFZ Potsdam Kp geomagnetic index. |
| 2. State Propagation | Both | Propagate the satellite state from one epoch to the next using the module's chosen dynamics model. |
| 3. Residual Analysis | Both | Compare predicted vs actual state; apply thresholds; classify detections as confirmed or uncertain based on Kp. |

The key difference between the two modules is the propagation model and data source:

- Kepler  uses analytical two-body Keplerian propagation. Fast, requires no Space-Track credentials. Best for detecting large manoeuvres such as altitude raises and plane changes.

- OD  uses numerical RK4 integration with J2 oblateness and atmospheric drag (US Standard Atmosphere 1976). Higher sensitivity; can detect small station-keeping manoeuvres. Requires a free Space-Track account.

# 2.  orbit_analysis_kepler.py

## 2.1  Data Source

Orbital element history is fetched from CelesTrak's public graph-orbit-data API. No account is required. Each row in the returned dataset contains one set of mean orbital elements at a given epoch.

Endpoint: https://celestrak.org/NORAD/elements/graph-orbit-data.php?CATNR={catnr}

Data is cached locally as a CSV file at:

data/orbit_{catnr}.csv

The cache is reused if the latest date in the file is within max_age_days (default: 1 day). If stale, the full dataset is re-fetched and the file is overwritten.

## 2.2  Propagation Model

For each consecutive epoch pair (i, i+1), the module propagates the Keplerian state at epoch i forward to epoch i+1 using a simple two-body analytical propagator:

### Step 1 — Solve Kepler's Equation

Given the mean anomaly M and eccentricity e at epoch i, the eccentric anomaly E is solved iteratively:

E₀ = M

Eₙ₊₁ = M + e · sin(Eₙ)   (100 iterations)

### Step 2 — Compute True Anomaly and Radius

ν  = 2 · arctan2( sqrt(1+e)·sin(E/2),  sqrt(1-e)·cos(E/2) )

r  = a · (1 - e·cos(E))

### Step 3 — Rotate to ECI Frame

The in-plane position (x_orb, y_orb) is rotated to Earth-Centred Inertial (ECI) coordinates using the classical rotation matrix parameterised by inclination i, RAAN Ω, and argument of perigee ω.

### Step 4 — Advance Mean Anomaly

To propagate forward by dt seconds, the mean motion n = sqrt(μ/a³) is used to advance the mean anomaly before repeating steps 1–3:

M_propagated = M + n · dt   (mod 2π)

Note: This is a two-body propagator — it ignores J2 oblateness, atmospheric drag, solar radiation pressure, and all other perturbations. The position error therefore grows with time, and propagation gaps larger than 10 days are discarded.

## 2.3  Background Drift Removal

Raw SMA and eccentricity deltas between epochs are noisy due to TLE fitting errors and the secular effects of J2 and atmospheric drag. To isolate genuine manoeuvre signatures, a local background drift rate is estimated and subtracted.

For each epoch pair centred at time t, a ±3 day window of smoothed SMA values is collected. A linear fit (degree-1 polynomial) gives the background drift rate in km/day. The excess delta SMA is then:

excess_delta_sma = delta_sma - (drift_rate_sma × gap_days)

Before drift removal, element histories are smoothed using a rolling(3, center=True).median() to suppress high-frequency TLE noise.

## 2.4  Manoeuvre Classification

A manoeuvre is flagged when the background-corrected change exceeds either threshold:

| Signal | Trigger Condition |
| --- | --- |
| excess_delta_sma | Excess SMA change > delta_sma_threshold km (default: 0.1 km) |
| delta_ecc | Eccentricity change > delta_ecc_threshold (default: 5×10⁻⁵) |

Flagged epochs are then cross-checked against the Kp space weather index:

| Classification | Condition |
| --- | --- |
| confirmed_maneuver | likely_maneuver AND Kp < kp_threshold (default: 5.0) |
| uncertain_maneuver | likely_maneuver AND Kp ≥ kp_threshold — geomagnetic storm may have caused a drag enhancement mimicking a manoeuvre |

## 2.5  Function Signature

def analyse_maneuvers_kepler(

catnr:               int,

data_dir:            str   = './data',

output_dir:          str   = './output',

max_age_days:        int   = 1,

delta_sma_threshold: float = 1.0,

delta_ecc_threshold: float = 5e-5,

kp_threshold:        float = 5.0,

) -> pd.DataFrame

| Parameter | Description |
| --- | --- |
| catnr | NORAD catalog number of the target satellite. |
| data_dir | Directory for cached CelesTrak CSV and Kp files. Created automatically if absent. |
| output_dir | Directory for output plot PNG files. Created automatically if absent. |
| max_age_days | Maximum age (days) of local CelesTrak cache before re-fetching. Default: 1. |
| delta_sma_threshold | Minimum excess SMA change (km) to flag a manoeuvre. Default: 1.0 km. Use 0.1 km for high sensitivity. |
| delta_ecc_threshold | Minimum eccentricity change to flag a manoeuvre. Default: 5×10⁻⁵. |
| kp_threshold | Kp value above which space weather is considered disturbed. Detections during disturbed periods are classified as uncertain. Default: 5.0. |

## 2.6  Output DataFrame

| Column | Type | Description |
| --- | --- | --- |
| epoch_from | Timestamp | Start epoch of the pair. |
| epoch_to | Timestamp | End epoch of the pair. |
| gap_days | float | Time gap between epochs in days. |
| error_km | float | Euclidean position error between predicted and actual position (km). |
| delta_sma | float | Smoothed SMA change across the pair (km). |
| excess_delta_sma | float | SMA change minus background drift (km). Primary detection signal. |
| drift_rate_sma | float | Local linear background drift rate (km/day) from ±3-day window. |
| delta_ecc | float | Smoothed eccentricity change. Secondary detection signal. |
| delta_inc | float | Smoothed inclination change (degrees). |
| kp | float | Kp geomagnetic index at epoch_from. |
| bad_space_weather | bool | True if Kp ≥ kp_threshold. |
| likely_maneuver | bool | True if excess_delta_sma or delta_ecc threshold exceeded. |
| confirmed_maneuver | bool | likely_maneuver AND low Kp. |
| uncertain_maneuver | bool | likely_maneuver AND high Kp. |

## 2.7  Output Plot

A four-panel PNG is saved to output_dir/kepler_maneuver_detection_{catnr}.png:

- Panel 1 — ΔSMA: raw smoothed delta and excess delta-SMA with threshold lines; confirmed/uncertain markers.

- Panel 2 — ΔEccentricity: smoothed eccentricity change with threshold lines.

- Panel 3 — Position Error: Euclidean distance between propagated and actual position (km).

- Panel 4 — Kp Index: geomagnetic index with threshold line and shaded disturbed regions.

# 3.  orbit_analysis_od.py

## 3.1  Data Source

TLE (Two-Line Element) history is downloaded from Space-Track.org, which provides archival TLEs for all tracked objects. A free account is required.

Endpoint: https://www.space-track.org/basicspacedata/query/class/gp_history/NORAD_CAT_ID/{catnr}/orderby/TLE_LINE1 ASC/format/tle

Data is cached locally at:

data/tle_history_{catnr}.txt

The cache is updated incrementally: on each run, the latest TLE epoch in the local file is checked. If it is less than 1 day old, the file is used as-is. Otherwise, only TLEs newer than the latest cached epoch are fetched from Space-Track and appended — avoiding repeated large downloads.

## 3.2  TLE Parsing and B* Extraction

Each TLE set is parsed to extract mean orbital elements in SGP4 mean-element theory. A key parameter is the B* drag term:

# TLE line 1, columns 54-61 format: SMMMMM±EE

# S = sign, MMMMM = mantissa×10⁻⁵, EE = exponent

B* = sign × mantissa × 10^exponent   [1/Earth-radius]

B* is used in three ways within the OD module: as a drag calibration input for the propagator, as a noise filter to exclude poorly-fitted TLEs, and as a secondary manoeuvre signal via step-changes in its value.

## 3.3  SGP4 State Conversion

TLE mean elements are not Keplerian osculating elements — they contain embedded secular and short-period SGP4 correction terms. Converting them directly to a position vector introduces errors of several km and tens of m/s in velocity. The correct approach is to run SGP4 at t = 0 (the TLE epoch) to obtain the osculating ECI state.

The module supports two SGP4 backends, selected automatically:

| Backend | Description |
| --- | --- |
| python-sgp4 (preferred) | Brandon Rhodes' python-sgp4 library (pip install sgp4). Implements full SGP4/SDP4 with automatic near-Earth / deep-space selection (Vallado 2006). Highest accuracy. |
| Built-in fallback | Hand-rolled SGP4 implementation (Hoots & Roehrich 1980). Near-Earth orbits only (period < 225 min). Used automatically when python-sgp4 is not installed. |

Recommendation: Install python-sgp4 for production use: pip install sgp4

## 3.4  RK4 Propagation with J2 + Drag

Once an osculating ECI state [r, v] is obtained from SGP4 at epoch i, the state is propagated forward to epoch i+1 using a 4th-order Runge-Kutta (RK4) integrator with the following force model:

### Two-body gravity

a_grav = -(μ/r³) · r_vec

### J2 oblateness perturbation

fac  = 1.5 · J2 · μ · Rₑ² / r⁵

a_J2 = fac · [ x·(5z²/r² - 1),  y·(5z²/r² - 1),  z·(5z²/r² - 3) ]

### Atmospheric drag

The atmosphere co-rotates with Earth, so drag is computed relative to the rotating atmosphere. The relative velocity is:

v_rel = [ vx + Ω_E·y,   vy - Ω_E·x,   vz ]

a_drag = -0.5 · CD_Am_eff · ρ(alt) · |v_rel| · v_rel

Atmospheric density ρ is computed from the US Standard Atmosphere 1976 piecewise exponential model with 28 altitude layers from 0 to 1000 km — significantly more accurate than a single-scale-height approximation above 100 km.

### Drag coefficient calibration

When use_bstar=True (the default), the effective drag parameter CD_Am_eff is derived from the TLE's B* value:

# B* [1/ER] → CD × Am [m²/kg]

# ρ₀ = 2.461×10⁻⁵ kg/m²/ER  (SGP4 reference density)

CD_Am_eff = 2 × |B*| / ρ₀

When use_bstar=False, the generic fallback CD × Am is used for all epochs.

### Integration

The RK4 step size is dt / max(10, dt/60) — at least 10 steps, with steps no larger than 1 minute. This balances accuracy against runtime for typical TLE gaps of a few hours to a few days.

## 3.5  Residual Computation

For each TLE pair (i, i+1):

- SGP4 at epoch i gives the initial osculating state [r₀, v₀].

- RK4 propagates [r₀, v₀] forward by gap_sec seconds → [r_pred, v_pred].

- SGP4 at epoch i+1 (dt=0) gives the actual state [r_actual, v_actual].

- Residuals: pos_residual_km = |r_pred − r_actual|,  vel_residual_ms = |v_pred − v_actual| × 1000.

Semi-major axes are derived from the vis-viva equation: a = 1 / (2/|r| − |v|²/μ), which converts the osculating state to an osculating SMA — a more physically meaningful quantity than SGP4 mean SMA.

## 3.6  Adaptive Threshold

Unlike a fixed threshold, the OD module derives its detection threshold from the data itself, making it self-calibrating to each satellite's noise floor:

### Gap normalisation

Velocity residuals grow roughly linearly with gap length due to integrator and atmosphere model errors. The residual is normalised by the gap length:

vel_res_per_day = vel_residual_ms / gap_days   [m/s/day]

### Robust statistics

The detection threshold is set at median + N × MAD of the finite gap-normalised residuals, where MAD is the Median Absolute Deviation — a robust spread estimator that is not inflated by the manoeuvre anomalies themselves:

median_vr         = median(vel_res_per_day)

mad_vr            = median(|vel_res_per_day - median_vr|)

threshold_per_day = median_vr + sigma_multiplier × mad_vr

Non-finite residuals (degenerate propagations) are excluded from the threshold computation and flagged separately in the vel_res_inf column.

Tuning: Increase sigma_multiplier (default: 5.0) to reduce false positives on satellites with noisy TLE histories. Decrease it to raise sensitivity for quiet satellites.

## 3.7  B* Step-Change Signal

In addition to the OD velocity residual, the module uses B* step-changes as a secondary corroborating signal. A sudden change in B* between consecutive TLEs indicates that the orbit fitter had to absorb an unmodelled force — often a manoeuvre:

delta_bstar = B*ᵢ₊₁ − B*ᵢ

bstar_flag  = |delta_bstar| > delta_bstar_threshold   (default: 5×10⁻⁵)

A TLE is flagged as a manoeuvre candidate if either the OD signal or the B* signal fires:

likely_maneuver = od_flag OR bstar_flag

## 3.8  B* Noise Filter

TLEs with unusually large |B*| values indicate a poor orbit fit — the fitter has over-fitted the drag term to compensate for errors elsewhere. Such TLEs are excluded before analysis:

# Skip TLEs where |B*| > bstar_noise_max (default: 1e-3)

This prevents a small number of badly-fitted TLEs from corrupting both the propagation and the adaptive threshold.

## 3.9  Manoeuvre Classification

The same Kp cross-check as the Kepler module is applied:

| Classification | Condition |
| --- | --- |
| confirmed_maneuver | likely_maneuver AND Kp < kp_threshold |
| uncertain_maneuver | likely_maneuver AND Kp ≥ kp_threshold |

## 3.10  Function Signature

def analyse_maneuvers_od(

catnr:                 int,

username:              str,

password:              str,

data_dir:              str   = './data',

output_dir:            str   = './output',

sigma_multiplier:      float = 5.0,

kp_threshold:          float = 5.0,

max_gap_days:          float = 10.0,

CD:                    float = 2.2,

Am:                    float = 0.01,

use_bstar:             bool  = True,

bstar_noise_max:       float = 1e-3,

delta_bstar_threshold: float = 5e-5,

) -> pd.DataFrame

| Parameter | Description |
| --- | --- |
| catnr | NORAD catalog number. |
| username | Space-Track login email. |
| password | Space-Track password. |
| data_dir | Directory for cached TLE and Kp files. |
| output_dir | Directory for output plot files. |
| sigma_multiplier | Adaptive threshold multiplier N in: threshold = median + N × MAD. Default: 5.0. |
| kp_threshold | Kp above which space weather is disturbed. Default: 5.0. |
| max_gap_days | Discard TLE pairs with epoch gap larger than this value. Default: 10.0 days. |
| CD | Fallback drag coefficient when B* is unavailable or use_bstar=False. Default: 2.2. |
| Am | Fallback area-to-mass ratio m²/kg when B* unavailable. Default: 0.01. |
| use_bstar | Use B*-derived drag in the RK4 propagator. Recommended. Default: True. |
| bstar_noise_max | Exclude TLEs where ¦B*¦ exceeds this value (bad orbit fits). Default: 1×10⁻³. |
| delta_bstar_threshold | Minimum ¦ΔB*¦ to fire the B* step-change signal. Default: 5×10⁻⁵. |

## 3.11  Output DataFrame

| Column | Type | Description |
| --- | --- | --- |
| epoch_from | Timestamp | Start epoch of the TLE pair. |
| epoch_to | Timestamp | End epoch of the TLE pair. |
| gap_days | float | Time gap between epochs (days). |
| sma | float | Osculating SMA at epoch_from from vis-viva equation (km). |
| delta_sma | float | Signed change in osculating SMA (km). |
| abs_delta_sma | float | Absolute SMA change (km). |
| delta_ecc | float | Change in mean eccentricity. |
| delta_inc | float | Change in mean inclination (degrees). |
| delta_raan | float | Change in RAAN, wrapped to ±180° (degrees). |
| bstar | float | B* drag term at epoch_from (1/Earth-radius). |
| delta_bstar | float | B* step-change across the pair. |
| pos_residual_km | float | Position residual ¦r_pred − r_actual¦ (km). |
| vel_residual_ms | float | Velocity residual ¦v_pred − v_actual¦ (m/s). |
| vel_res_per_day | float | Gap-normalised velocity residual (m/s/day). Primary detection signal. |
| kp | float | Kp geomagnetic index at epoch_from. |
| bad_space_weather | bool | True if Kp ≥ kp_threshold. |
| od_flag | bool | True if vel_res_per_day exceeds adaptive threshold. |
| bstar_flag | bool | True if ¦delta_bstar¦ exceeds delta_bstar_threshold. |
| vel_res_inf | bool | True if vel_residual was non-finite (degenerate propagation). |
| likely_maneuver | bool | od_flag OR bstar_flag. |
| confirmed_maneuver | bool | likely_maneuver AND low Kp. |
| uncertain_maneuver | bool | likely_maneuver AND high Kp. |

## 3.12  Output Plot

A seven-panel PNG is saved to output_dir/od_maneuver_detection_{catnr}.png:

- Panel 1 — Gap-normalised velocity residual: vel_res_per_day with adaptive threshold line (log scale).

- Panel 2 — Raw velocity residual: vel_residual_ms in m/s (log scale). Reference panel.

- Panel 3 — B* history: absolute B* value over time.

- Panel 4 — ΔB*: B* step-changes with threshold lines.

- Panel 5 — ΔSMA: osculating SMA change (km).

- Panel 6 — ΔEccentricity: mean eccentricity change.

- Panel 7 — Kp Index: geomagnetic index with threshold and shaded disturbed regions.

All panels share a common x-axis (epoch) and mark confirmed manoeuvres in red and uncertain manoeuvres in orange.

# 4.  Shared: Kp Geomagnetic Index

Both modules use the same Kp caching and lookup functions. The Kp index is a 3-hourly planetary geomagnetic activity index (0–9) published by GFZ Potsdam.

## 4.1  Data Source and Caching

Sources: Nowcast: https://kp.gfz-potsdam.de/app/files/Kp_ap_nowcast.txt ¦ Historical: https://kp.gfz-potsdam.de/app/files/Kp_ap_Ap_SN_F107_since_1932.txt

Both the raw text and parsed CSV are cached locally:

data/kp_index.txt   # raw GFZ download

data/kp_index.csv   # parsed DataFrame

Caching strategy:

- First run: downloads the full historical file (1932–present, ~30 MB).

- Subsequent runs: if the latest timestamp is more than 1 day old, only the compact nowcast file is fetched and merged — avoiding repeated large downloads.

- Corrupt file detection: if any Kp value falls outside 0–9, the file is deleted and re-downloaded from scratch.

## 4.2  Epoch Lookup

Each epoch is matched to the 3-hour Kp interval that contains it. If the epoch falls outside the available Kp data range (e.g. very recent TLEs before the nowcast is updated), the nearest available interval is used as a fallback.

# 5.  Method Comparison

| Property | Kepler | OD |
| --- | --- | --- |
| Data source | CelesTrak (public) | Space-Track (free account) |
| Credentials required | None | Space-Track email + password |
| Propagation model | Two-body Keplerian | SGP4 + RK4 (J2 + drag) |
| Drag model | None | US Std Atm 1976 + B* calibration |
| Detection signal | ΔSMA, ΔEcc (element deltas) | Velocity residual + ΔB* |
| Threshold type | Fixed (user-supplied) | Adaptive (median + N×MAD) |
| Sensitivity | Lower — large manoeuvres | Higher — small manoeuvres |
| Runtime | Fast (no integration) | Slower (per-pair RK4) |
| Output panels | 4 | 7 |
| Recommended use | Quick screening | Detailed confirmation |

## 5.1  Recommended Workflow

For a new satellite, the recommended workflow is:

- Step 1 — Kepler screening: run the Kepler method first. It requires no credentials, runs in seconds, and immediately identifies obvious altitude-raising or plane-change events in the SMA and eccentricity plots.

- Step 2 — OD confirmation: run the OD method on the same satellite. Cross-reference the detected epochs. Manoeuvres confirmed by both methods with consistent epoch ranges are high-confidence detections.

- Step 3 — Uncertain review: examine epochs flagged as uncertain (high Kp). Check whether the Kp disturbance is short-lived and whether multiple consecutive epochs are flagged — a true manoeuvre typically produces a localised spike rather than a broad Kp-correlated elevation.

# 6.  Constants and Physical Parameters

| Constant | Value and Description |
| --- | --- |
| μ (MU) | 398600.4418 km³/s² — Earth gravitational parameter (WGS-84). |
| J2 | 1.08263×10⁻³ — Earth's second zonal harmonic (oblateness coefficient). |
| Rₑ (RE) | 6378.137 km — Earth equatorial radius (WGS-84). |
| Ω_E (OMEGA_E) | 7.2921150×10⁻⁵ rad/s — Earth rotation rate. |
| ρ₀ (SGP4) | 2.461×10⁻⁵ kg/m²/ER — SGP4 reference atmospheric density used for B* conversion. |
| Atmosphere | US Standard Atmosphere 1976, piecewise exponential, 28 layers from 0–1000 km. |

# 7.  Dependencies

| Package | Purpose |
| --- | --- |
| numpy | Numerical arrays, linear algebra, and trigonometry. |
| pandas | DataFrame operations, time-series handling, CSV I/O. |
| matplotlib | Output plots (PNG). |
| requests | HTTP downloads from CelesTrak, Space-Track, and GFZ Potsdam. |
| sgp4 | Optional (pip install sgp4). Enables full SGP4/SDP4 via python-sgp4. Without it, the built-in near-Earth SGP4 fallback is used automatically. |

pip install numpy pandas matplotlib requests

pip install sgp4   # optional but recommended

Python version: 3.10+ required (uses list[dict] and dict[str, ...] type hints in function signatures).